.: CURRENT : ELECTRICITY : LAB :.

Purpose:

To gain an understanding of the relationship between current and potential difference in series and parallel circuits.  To determine whether resistors are Ohmic.

Task:

Working in groups of 2-3, you will build a variety of circuits and use meters to measure the current & potential difference.

Materials:

- 3 batteries

- 1 battery holder

- 2 light bulbs

- 1 light bulb holder

- 1 resistor plate

- 10-12 wires

- 1 ammeter

- 1 voltmeter

.: PART A – SERIES & PARALLEL CIRCUITS :.

Instructions

- Build a circuit with 1 battery and 1 light bulb.

- Describe the brightness of the bulb on a scale of 1-5 (5 being very bright).

- Determine the current and potential difference of the circuit.  Record it below.

- Repeat steps 1-3 with a 2nd and 3rd battery.

- Based on your answers from the 3-battery circuit, predict the total current and potential difference if you connect a second light bulb in series and in parallel.  Also predict the brightness of the bulbs.  Record your predictions in the chart.

- Add a second load in series to your circuit. Record the relative brightness of each bulb.

- Determine and record the actual current and potential difference of the entire circuit.

- Change your circuit so that the two loads are in parallel.  Draw the schematic diagram of this circuit.  Include an ammeter and voltmeter measuring the entire circuit[1]

- Record the relative brightness of each bulb.

- Determine and record the actual current and potential difference of the entire circuit.

- Dismantle your circuit. Clean up.

Observation chart

| Circuit | Current | Potential Difference | Brightness of bulbs (1 = dim, 5 = bright) |
| --- | --- | --- | --- |
| 1 bulb, 1 battery |  |  |  |
| 1 bulb, 2 batteries |  |  |  |
| 1 bulb, 3 batteries |  |  |  |
| 2 bulbs in series, 3 batteries | Predicted | Predicted | Predicted |
|  | Actual | Actual | Actual |
| 2 bulbs in parallel, 3 batteries | Predicted | Predicted | Predicted |
|  | Actual | Actual | Actual |

.: PART B - RESISTORS :.

Instructions

- Obtain a resistor from your teacher.  Connect the resistor to a battery.

- Using the ammeter and voltmeter, obtain a reading for the current and potential difference.  Record the data on the observation chart.

- Repeat the above steps with 2, 3 and 4 batteries.

- Dismantle your circuit.  Clean up.

Observation chart

| Number of batteries | Current | Potential Difference |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |

# .: ANALYSIS :.

- Convert values from observation chart in Part B into appropriate SI units.

- Plot the values of the current and potential difference from Part B.  Place the values on the appropriate axis so that resistance can be measured from the value of the slope.  Remember to properly label the axes and title the graph. Draw a line of best fit through the points.[5]

- Calculate the resistance of the load using the graph.  Indicate the points on the graph that you used for the calculation (i.e. make the rise/run triangle).  Show your calculations on the graph.

- Is the resistor Ohmic?  Explain by giving evidence for your conclusion.

- Often the actual values of the current and potential difference are not exactly the same as the predicted theoretical values.  What are two assumptions that are made when we make predictions about these values?  (hint: what is assumed to be constant?)